割草机器人 RK3588 + ROS1 Noetic 工程包优化方案

# 割草机器人工程包优化与整合方案（RK3588 + ROS1 Noetic）

## 一、工程整体架构梳理

### 核心依赖关系

Plain Text src/ ├── rk3588_sdk/ # 底层硬件抽象层（串口/485/GPIO/ARM64适配） ├── motor_control/ # 差速电机控制（Serial协议+轮速回采） ├── ultrasonic_sensor/ # 12路超声波雷达（485通信） ├── orbbec_camera/ # 奥比中光Gemini335（USB通信） ├── hesai_lidar/ # 禾赛JT128激光雷达（ETH通信） ├── handsfree_rtk/ # UM982 RTK定位（USB通信） ├── lawn_mower/ # 整合包（launch/参数配置/整体调度） └── CMakeLists.txt # 工作空间根配置（ARM64编译优化）

## 二、核心模块配置优化

### 1. rk3588_sdk 优化（底层驱动核心）

#### 1.1 CMakeLists.txt（src/rk3588_sdk/CMakeLists.txt）

CMake cmake_minimum_required(VERSION 3.10) project(rk3588_sdk) # 引入RK3588 ARM64工具链 include(${CMAKE_SOURCE_DIR}/../../rk3588_toolchain.cmake) # ROS1 Noetic依赖 find_package(catkin REQUIRED COMPONENTS roscpp serial std_msgs ) # 导出库和头文件 catkin_package( INCLUDE_DIRS include LIBRARIES rk3588_sdk CATKIN_DEPENDS roscpp serial std_msgs DEPENDS system_lib ) # 包含目录（ARM64系统头文件+ROS头文件） include_directories( include ${catkin_INCLUDE_DIRS} /usr/include/aarch64-linux-gnu # RK3588系统头文件 ) # 编译核心驱动（串口+485+GPIO） add_library(${PROJECT_NAME} src/rk3588_serial.cpp src/rk3588_gpio.cpp src/uart_485.c src/gpio_control.cpp src/uart_driver.cpp ) # 链接依赖（ARM64串口/485库） target_link_libraries(${PROJECT_NAME} ${catkin_LIBRARIES} ${SERIAL_LIBRARIES} pthread # 485通信线程依赖 ) # 编译选项（ARM64优化） target_compile_options(${PROJECT_NAME} PRIVATE -march=armv8.2-a -O2 -Wall -fPIC ) # 安装规则 install(DIRECTORY include/ DESTINATION ${CATKIN_PACKAGE_INCLUDE_DESTINATION} ) install(TARGETS ${PROJECT_NAME} ARCHIVE DESTINATION ${CATKIN_PACKAGE_LIB_DESTINATION} LIBRARY DESTINATION ${CATKIN_PACKAGE_LIB_DESTINATION} RUNTIME DESTINATION ${CATKIN_PACKAGE_BIN_DESTINATION} )

#### 1.2 package.xml（src/rk3588_sdk/package.xml）

XML <?xml version="1.0" encoding="UTF-8"?> <package format="2"> <name>rk3588_sdk</name> <version>1.0.0</version> <description>RK3588 SDK驱动包，适配ROS1 Noetic，为割草机器人提供串口/485/GPIO硬件底层支持</description> <maintainer email="norco@norco.com">norco</maintainer> <license>Apache-2.0</license> <buildtool_depend>catkin</buildtool_depend> <!-- ROS1核心依赖 --> <depend>roscpp</depend> <depend>rospy</depend> <depend>std_msgs</depend> <depend>serial</depend> <!-- ARM64硬件依赖 --> <build_depend>libusb-1.0-dev</build_depend> <build_depend>libudev-dev</build_depend> <build_depend>libboost-all-dev</build_depend> <build_depend>libpthread-stubs0-dev</build_depend> <exec_depend>libusb-1.0-0</exec_depend> <exec_depend>libudev1</exec_depend> <exec_depend>libboost-all-dev</exec_depend> <export> <catkin_libraries>rk3588_hardware</catkin_libraries> <include_directories>include</include_directories> </export> </package>

### 2. orbbec_camera 优化（Gemini335 USB版）

#### 2.1 CMakeLists.txt（src/orbbec_camera/CMakeLists.txt）

CMake cmake_minimum_required(VERSION 3.10.2) project(orbbec_camera) # ROS1 Noetic + ARM64编译配置 set(CMAKE_CXX_STANDARD 17) set(CMAKE_C_STANDARD 17) set(CMAKE_BUILD_TYPE "Release") set(CMAKE_CXX_FLAGS "${CMAKE_CXX_FLAGS} -march=armv8.2-a -O2 -Wall -fPIC") set(CMAKE_C_FLAGS "${CMAKE_C_FLAGS} -march=armv8.2-a -O2 -Wall -fPIC") # 启用RK3588硬件解码（适配Gemini335） option(USE_RK_HW_DECODER "Use Rockchip hardware decoder" ON) option(USE_NV_HW_DECODER "Use Nvidia hardware decoder" OFF) # 检测RK3588 ARM64架构 execute_process(COMMAND uname -m OUTPUT_VARIABLE MACHINES) execute_process(COMMAND getconf LONG_BIT OUTPUT_VARIABLE MACHINES_BIT) if ((${MACHINES} MATCHES "aarch64") AND (${MACHINES_BIT} MATCHES "64")) set(HOST_PLATFORM "arm64") endif () # ROS1依赖 find_package(catkin REQUIRED COMPONENTS roscpp sensor_msgs cv_bridge image_transport camera_info_manager tf2_ros nodelet rk3588_sdk # 依赖RK3588底层SDK ) # 导出配置 catkin_package( INCLUDE_DIRS include SDK/include LIBRARIES orbbec_camera CATKIN_DEPENDS roscpp sensor_msgs cv_bridge rk3588_sdk ) # 路径配置（适配RK3588 ARM64） set(ORBBEC_LIBS_DIR ${CMAKE_CURRENT_SOURCE_DIR}/SDK/lib/${HOST_PLATFORM}) set(ORBBEC_INCLUDE_DIR ${CMAKE_CURRENT_SOURCE_DIR}/SDK/include/) link_directories(${ORBBEC_LIBS_DIR}) # 包含目录 include_directories( include ${ORBBEC_INCLUDE_DIR} ${catkin_INCLUDE_DIRS} ${CMAKE_SOURCE_DIR}/../rk3588_sdk/include # RK3588 SDK头文件 ) # 编译核心节点（Gemini335适配） add_executable(ob_camera_node src/ob_camera_node.cpp src/ob_camera_node_driver.cpp src/rk_mpp_decoder.cpp # RK硬件解码 src/jpeg_decoder.cpp src/utils.cpp src/ros_sensor.cpp ) # 链接依赖（ARM64 + RK3588 SDK） target_link_libraries(ob_camera_node ${catkin_LIBRARIES} rk3588_sdk ${ORBBEC_LIBS_DIR}/libObTypes.so ${ORBBEC_LIBS_DIR}/libObDevice.so ${ORBBEC_LIBS_DIR}/libObSensor.so ) # 安装规则 install(TARGETS ob_camera_node RUNTIME DESTINATION ${CATKIN_PACKAGE_BIN_DESTINATION} ) install(DIRECTORY launch/ config/ rviz/ SDK/lib/arm64/ DESTINATION ${CATKIN_PACKAGE_SHARE_DESTINATION} ) install(DIRECTORY include/ DESTINATION ${CATKIN_PACKAGE_INCLUDE_DESTINATION} )

#### 2.2 package.xml（src/orbbec_camera/package.xml）

XML <?xml version="1.0"?> <package format="2"> <!-- ROS1 Noetic使用format2 --> <name>orbbec_camera</name> <version>2.7.6</version> <description>Orbbec Gemini335 Camera package for RK3588 (ROS1 Noetic)</description> <maintainer email="mocun@orbbec.com">Joe Dong</maintainer> <license>Apache License 2.0</license> <buildtool_depend>catkin</buildtool_depend> <!-- 核心依赖 --> <depend>roscpp</depend> <depend>sensor_msgs</depend> <depend>cv_bridge</depend> <depend>image_transport</depend> <depend>camera_info_manager</depend> <depend>tf2_ros</depend> <depend>nodelet</depend> <depend>rk3588_sdk</depend> <!-- 依赖RK3588底层SDK --> <!-- ARM64硬件依赖 --> <build_depend>libusb-1.0-dev</build_depend> <build_depend>libudev-dev</build_depend> <exec_depend>libusb-1.0-0</exec_depend> <exec_depend>libudev1</exec_depend> <export> <nodelet plugin="${prefix}/nodelet_plugins.xml"/> </export> </package>

### 3. hesai_lidar 优化（JT128 ETH版）

#### 3.1 CMakeLists.txt（src/hesai_lidar/CMakeLists.txt）

CMake cmake_minimum_required(VERSION 3.10) project(hesai_lidar) # 引入RK3588工具链（禁用CUDA，RK3588不兼容） include(${CMAKE_SOURCE_DIR}/../../rk3588_toolchain.cmake) set(FIND_CUDA OFF) # 强制禁用CUDA # ROS1 Noetic依赖 find_package(catkin REQUIRED COMPONENTS roscpp std_msgs sensor_msgs message_generation rk3588_sdk ) # 生成ROS消息（仅ROS1） add_message_files( FILES Firetime.msg LossPacket.msg Ptp.msg UdpPacket.msg UdpFrame.msg ) generate_messages( DEPENDENCIES std_msgs sensor_msgs ) # 导出配置 catkin_package( INCLUDE_DIRS src/driver src/utility LIBRARIES hesai_lidar_lib CATKIN_DEPENDS roscpp std_msgs sensor_msgs message_runtime rk3588_sdk ) # 包含目录（ARM64适配） include_directories( ${catkin_INCLUDE_DIRS} src/utility src/manager src/driver src/driver/HesaiLidar_SDK_2.0 src/driver/HesaiLidar_SDK_2.0/libhesai ${CMAKE_SOURCE_DIR}/../rk3588_sdk/include ) # 编译SDK核心库（JT128适配） add_subdirectory(src/driver/HesaiLidar_SDK_2.0) # 编译禾赛雷达节点 add_executable(hesai_lidar_node src/driver/hesai_driver.cpp src/manager/lidar_manager.cpp src/utility/lidar_utils.cpp ) # 链接依赖（ARM64 + 禁用CUDA） target_link_libraries(hesai_lidar_node ${catkin_LIBRARIES} rk3588_sdk hesai_sdk_lib lidar_lib pthread ) # 编译选项（ARM64优化） target_compile_options(hesai_lidar_node PRIVATE -march=armv8.2-a -O2 -Wall -std=c++14 ) # 安装规则 install(TARGETS hesai_lidar_node RUNTIME DESTINATION ${CATKIN_PACKAGE_BIN_DESTINATION} ) install(DIRECTORY config launch rviz DESTINATION ${CATKIN_PACKAGE_SHARE_DESTINATION} )

#### 3.2 package.xml（src/hesai_lidar/package.xml）

XML <?xml version="1.0"?> <package format="2"> <!-- ROS1 Noetic使用format2 --> <name>hesai_lidar</name> <version>1.5.0</version> <description>Hesai JT128 LiDAR package for RK3588 (ROS1 Noetic)</description> <maintainer email="zhangyu@hesaitech.com">hesaiwuxiaozhou</maintainer> <license>BSD</license> <buildtool_depend>catkin</buildtool_depend> <!-- ROS1核心依赖 --> <build_depend>roscpp</build_depend> <build_depend>sensor_msgs</build_depend> <build_depend>std_msgs</build_depend> <build_depend>message_generation</build_depend> <exec_depend>roscpp</exec_depend> <exec_depend>sensor_msgs</exec_depend> <exec_depend>std_msgs</exec_depend> <exec_depend>message_runtime</exec_depend> <exec_depend>rk3588_sdk</exec_depend> <export> <build_type>catkin</build_type> </export> </package>

### 4. handsfree_rtk 优化（UM982 USB版）

#### 4.1 CMakeLists.txt（src/handsfree_rtk/CMakeLists.txt）

CMake cmake_minimum_required(VERSION 3.10.2) project(handsfree_rtk) # ARM64编译优化 set(CMAKE_CXX_STANDARD 17) set(CMAKE_CXX_FLAGS "${CMAKE_CXX_FLAGS} -march=armv8.2-a -O2 -Wall") # ROS1 Noetic依赖 find_package(catkin REQUIRED COMPONENTS geometry_msgs roscpp rospy sensor_msgs rk3588_sdk ) # 导出配置 catkin_package( INCLUDE_DIRS include CATKIN_DEPENDS roscpp rospy sensor_msgs rk3588_sdk ) # 包含目录 include_directories( include ${catkin_INCLUDE_DIRS} ${CMAKE_SOURCE_DIR}/../rk3588_sdk/include ) # 编译C++驱动（UM982 USB适配） add_executable(rtk_um982_node demo/c++/um982_driver.cpp ) target_link_libraries(rtk_um982_node ${catkin_LIBRARIES} rk3588_sdk # 依赖RK3588串口驱动 ) # 安装Python脚本（ROS1 Noetic Python3） catkin_install_python(PROGRAMS scripts/ros_driver_um98x.py scripts/ros_driver_um98x_ntrip.py DESTINATION ${CATKIN_PACKAGE_BIN_DESTINATION} ) # 安装规则 install(TARGETS rtk_um982_node RUNTIME DESTINATION ${CATKIN_PACKAGE_BIN_DESTINATION} ) install(DIRECTORY launch usb_rules/ DESTINATION ${CATKIN_PACKAGE_SHARE_DESTINATION} )

#### 4.2 package.xml（src/handsfree_rtk/package.xml）

XML <?xml version="1.0"?> <package format="2"> <name>handsfree_rtk</name> <version>1.0.0</version> <description>Handsfree UM982 RTK package for RK3588 (ROS1 Noetic)</description> <maintainer email="handsfree@norco.com">handsfree</maintainer> <license>Apache-2.0</license> <buildtool_depend>catkin</buildtool_depend> <!-- ROS1依赖 --> <build_depend>geometry_msgs</build_depend> <build_depend>roscpp</build_depend> <build_depend>rospy</build_depend> <build_depend>sensor_msgs</build_depend> <build_depend>rk3588_sdk</build_depend> <exec_depend>geometry_msgs</exec_depend> <exec_depend>roscpp</exec_depend> <exec_depend>rospy</exec_depend> <exec_depend>sensor_msgs</exec_depend> <exec_depend>rk3588_sdk</exec_depend> <!-- USB串口依赖 --> <build_depend>libusb-1.0-dev</build_depend> <exec_depend>libusb-1.0-0</exec_depend> <export> </export> </package>

### 5. motor_control 优化（差速电机+轮速回采）

#### 5.1 CMakeLists.txt（src/motor_control/CMakeLists.txt）

CMake cmake_minimum_required(VERSION 3.10) project(motor_control) # 引入RK3588工具链 include(${CMAKE_SOURCE_DIR}/../../rk3588_toolchain.cmake) # ROS1依赖 find_package(catkin REQUIRED COMPONENTS roscpp geometry_msgs std_msgs serial rk3588_sdk ) # 导出配置 catkin_package( INCLUDE_DIRS include CATKIN_DEPENDS roscpp geometry_msgs std_msgs serial rk3588_sdk ) # 包含目录 include_directories( include ${catkin_INCLUDE_DIRS} ${CMAKE_SOURCE_DIR}/../rk3588_sdk/include ) # 编译电机控制节点（Serial协议+485+轮速回采） add_executable(motor_control_node src/motor_node.cpp src/serial_protocol.cpp src/motor_driver.cpp src/pid_controller.cpp src/rs485_comm.cpp src/serial_port.cpp src/motor_controller.cpp ) # 链接依赖（RK3588 485/串口库） target_link_libraries(motor_control_node ${catkin_LIBRARIES} rk3588_sdk pthread ) # ARM64编译优化 target_compile_options(motor_control_node PRIVATE -march=armv8.2-a -O2 -Wall -std=c++17 ) # 安装规则 install(TARGETS motor_control_node RUNTIME DESTINATION ${CATKIN_PACKAGE_BIN_DESTINATION} ) install(DIRECTORY config launch DESTINATION ${CATKIN_PACKAGE_SHARE_DESTINATION} )

### 6. ultrasonic_sensor 优化（12路485）

#### 6.1 CMakeLists.txt（src/ultrasonic_sensor/CMakeLists.txt）

CMake cmake_minimum_required(VERSION 3.10.2) project(ultrasonic_sensor) # ARM64编译配置 set(CMAKE_CXX_STANDARD 17) set(CMAKE_CXX_FLAGS "${CMAKE_CXX_FLAGS} -march=armv8.2-a -O2 -Wall") # ROS1依赖 find_package(catkin REQUIRED COMPONENTS roscpp std_msgs sensor_msgs rk3588_sdk ) # 导出配置 catkin_package( INCLUDE_DIRS include CATKIN_DEPENDS roscpp std_msgs sensor_msgs rk3588_sdk ) # 包含目录 include_directories( include ${catkin_INCLUDE_DIRS} ${CMAKE_SOURCE_DIR}/../rk3588_sdk/include ) # 编译12路超声波节点 add_executable(ultrasonic_sensor_node src/ultrasonic_node.cpp src/ultrasonic_driver.cpp ) # 链接RK3588 485库 target_link_libraries(ultrasonic_sensor_node ${catkin_LIBRARIES} rk3588_sdk pthread ) # 安装规则 install(TARGETS ultrasonic_sensor_node RUNTIME DESTINATION ${CATKIN_PACKAGE_BIN_DESTINATION} ) install(DIRECTORY config launch DESTINATION ${CATKIN_PACKAGE_SHARE_DESTINATION} )

## 三、整合包 lawn_mower（核心launch+参数配置）

### 3.1 目录结构

Plain Text src/lawn_mower/ ├── launch/ │ ├── lawn_mower.launch # 总启动文件 │ ├── sensors.launch # 传感器启动子文件 │ └── motor_control.launch # 电机控制启动子文件 ├── config/ │ ├── motor_params.yaml # 电机PID/串口参数 │ ├── sensor_params.yaml # 传感器标定参数 │ └── rtk_params.yaml # RTK配置参数 ├── CMakeLists.txt └── package.xml

### 3.2 总启动文件（launch/lawn_mower.launch）

XML <launch> <!-- 启动RK3588 SDK节点 --> <node name="rk3588_sdk_node" pkg="rk3588_sdk" type="sdk_node" output="screen"/> <!-- 启动传感器（相机+雷达+RTK） --> <include file="$(find lawn_mower)/launch/sensors.launch"/> <!-- 启动电机控制+超声波 --> <include file="$(find lawn_mower)/launch/motor_control.launch"/> <!-- RVIZ可视化（可选） --> <node name="rviz" pkg="rviz" type="rviz" args="-d $(find lawn_mower)/config/lawn_mower.rviz" output="screen"/> </launch>

### 3.3 传感器启动文件（launch/sensors.launch）

XML <launch> <!-- 奥比中光Gemini335相机（USB） --> <node name="orbbec_camera_node" pkg="orbbec_camera" type="ob_camera_node" output="screen"> <param name="camera/device_type" value="gemini335"/> <param name="camera/usb_port" value="/dev/ttyACM0"/> <param name="camera/frame_id" value="camera_link"/> <rosparam file="$(find lawn_mower)/config/sensor_params.yaml" command="load"/> </node> <!-- 禾赛JT128激光雷达（ETH） --> <node name="hesai_lidar_node" pkg="hesai_lidar" type="hesai_lidar_node" output="screen"> <param name="lidar/type" value="JT128"/> <param name="lidar/ip" value="192.168.1.200"/> <param name="lidar/port" value="2368"/> <param name="lidar/frame_id" value="lidar_link"/> <rosparam file="$(find lawn_mower)/config/sensor_params.yaml" command="load"/> </node> <!-- UM982 RTK（USB） --> <node name="rtk_um982_node" pkg="handsfree_rtk" type="ros_driver_um98x.py" output="screen"> <param name="rtk/usb_port" value="/dev/ttyUSB0"/> <param name="rtk/baudrate" value="115200"/> <param name="rtk/frame_id" value="rtk_link"/> <rosparam file="$(find lawn_mower)/config/rtk_params.yaml" command="load"/> </node> <!-- 12路超声波雷达（485） --> <node name="ultrasonic_sensor_node" pkg="ultrasonic_sensor" type="ultrasonic_sensor_node" output="screen"> <param name="ultrasonic/485_port" value="/dev/ttyS4"/> <param name="ultrasonic/baudrate" value="9600"/> <param name="ultrasonic/num_sensors" value="12"/> <rosparam file="$(find lawn_mower)/config/sensor_params.yaml" command="load"/> </node> </launch>

### 3.4 电机控制启动文件（launch/motor_control.launch）

XML <launch> <!-- 差速电机控制（Serial+轮速回采） --> <node name="motor_control_node" pkg="motor_control" type="motor_control_node" output="screen"> <!-- 串口参数 --> <param name="motor/serial_port" value="/dev/ttyS2"/> <param name="motor/baudrate" value="115200"/> <param name="motor/parity" value="none"/> <!-- 电机参数 --> <param name="motor/pid/kp" value="0.8"/> <param name="motor/pid/ki" value="0.1"/> <param name="motor/pid/kd" value="0.05"/> <param name="motor/max_speed" value="1.0"/> <!-- m/s --> <!-- 轮速回采 --> <param name="motor/odom/rate" value="50"/> <!-- Hz --> <rosparam file="$(find lawn_mower)/config/motor_params.yaml" command="load"/> </node> </launch>

### 3.5 电机参数配置（config/motor_params.yaml）

YAML motor: left_wheel: pid: kp: 0.8 ki: 0.1 kd: 0.05 max_speed: 1.0 # 最大线速度 m/s encoder_resolution: 1024 # 编码器分辨率 right_wheel: pid: kp: 0.8 ki: 0.1 kd: 0.05 max_speed: 1.0 encoder_resolution: 1024 serial: port: /dev/ttyS2 baudrate: 115200 timeout: 0.1 odom: publish_rate: 50 # Hz wheel_base: 0.5 # 轮距 m wheel_radius: 0.1 # 轮径 m

### 3.6 lawn_mower 包配置

#### CMakeLists.txt（src/lawn_mower/CMakeLists.txt）

CMake cmake_minimum_required(VERSION 3.10.2) project(lawn_mower) find_package(catkin REQUIRED COMPONENTS roscpp rospy rk3588_sdk orbbec_camera hesai_lidar handsfree_rtk motor_control ultrasonic_sensor ) catkin_package( CATKIN_DEPENDS roscpp rospy rk3588_sdk orbbec_camera hesai_lidar handsfree_rtk motor_control ultrasonic_sensor ) include_directories( ${catkin_INCLUDE_DIRS} ) # 安装launch和配置文件 install(DIRECTORY launch/ config/ DESTINATION ${CATKIN_PACKAGE_SHARE_DESTINATION} )

#### package.xml（src/lawn_mower/package.xml）

XML <?xml version="1.0"?> <package format="2"> <name>lawn_mower</name> <version>1.0.0</version> <description>Lawn Mower Robot Integration Package (RK3588 + ROS1 Noetic)</description> <maintainer email="dev@norco.com">Norco Dev</maintainer> <license>MIT</license> <buildtool_depend>catkin</buildtool_depend> <!-- 依赖所有子包 --> <depend>rk3588_sdk</depend> <depend>orbbec_camera</depend> <depend>hesai_lidar</depend> <depend>handsfree_rtk</depend> <depend>motor_control</depend> <depend>ultrasonic_sensor</depend> <!-- ROS1核心依赖 --> <depend>roscpp</depend> <depend>rospy</depend> <depend>rviz</depend> <export> <build_type>catkin</build_type> </export> </package>

## 四、编译与调试说明

### 1. 环境准备（RK3588 + Ubuntu20.04 + ROS1 Noetic）

Bash # 安装依赖 sudo apt install ros-noetic-catkin ros-noetic-serial ros-noetic-cv-bridge ros-noetic-image-transport ros-noetic-pcl-ros sudo apt install libusb-1.0-dev libudev-dev libboost-all-dev # 设置环境变量 echo "source /opt/ros/noetic/setup.bash" >> ~/.bashrc echo "source ~/catkin_ws/devel/setup.bash" >> ~/.bashrc source ~/.bashrc

### 2. 编译工程

Bash cd ~/catkin_ws catkin_make -DCMAKE_BUILD_TYPE=Release -DCATKIN_ENABLE_TESTING=0

### 3. 启动割草机器人

Bash # 启动所有节点 roslaunch lawn_mower lawn_mower.launch # 单独启动传感器 roslaunch lawn_mower sensors.launch # 单独启动电机控制 roslaunch lawn_mower motor_control.launch

### 4. 关键调试指令

Bash # 查看电机话题 rostopic echo /motor/odom # 查看超声波数据 rostopic echo /ultrasonic/distance # 查看RTK定位 rostopic echo /rtk/fix # 查看雷达点云 rostopic echo /hesai/points # 查看相机图像 rviz -d $(find lawn_mower)/config/lawn_mower.rviz

## 五、关键优化点说明

- RK3588 ARM64适配：所有CMakeLists添加-march=armv8.2-a编译选项，适配RK3588架构；

- 依赖统一：所有模块依赖rk3588_sdk，统一硬件抽象层，避免重复驱动开发；

- 通信协议适配：

- Orbbec相机：USB通信，启用RK3588硬件解码；

- 禾赛雷达：ETH通信，禁用CUDA适配ARM64；

- RTK/电机/超声波：复用RK3588 SDK的串口/485驱动；

- ROS1规范：所有package.xml改为format2，移除ROS2条件编译，适配Noetic；

- 参数解耦：将硬件参数抽离到yaml文件，便于调试；

- 启动整合：通过launch文件分层启动，便于单独调试各模块。

此方案保证了所有模块在RK3588 + Ubuntu20.04 + ROS1 Noetic环境下的编译兼容性，且具备完整的割草机器人传感器+运动控制功能，可直接基于此结构进行二次开发。