Lawnwomer_ws 割草机器人工程解析与编译优化

# 割草机器人lawnwomer_ws工程全解析与编译优化

## 一、工程文件结构与包含关系梳理

### 1. 整体目录结构

Plain Text lawnwomer-ws/ ├── src/ │ ├── HesaiLidar_ROS/ # 禾赛JT128激光雷达ROS驱动（ETH通信） │ ├── HesaiLidar_SDK/ # 禾赛雷达底层SDK（被HesaiLidar_ROS依赖） │ ├── handsfree_rtk/ # T-RTK UM982相关（USB通信，待完善） │ ├── OrbbecSDK_ROS/ # 奥比中光相机（暂未用到，保留） │ └── CMakeLists.txt # catkin顶层构建文件 ├── build/ # 编译目录（自动生成） ├── devel/ # 开发目录（自动生成） └── install/ # 安装目录（自动生成）

### 2. 核心模块依赖关系

| 模块 | 通信方式 | 依赖项 | 被依赖方 |
| --- | --- | --- | --- |
| HesaiLidar_ROS | ETH | HesaiLidar_SDK、ROS1、yaml-cpp | 顶层catkin工程 |
| HesaiLidar_SDK | ETH | CUDA（可选）、OpenSSL、PCL | HesaiLidar_ROS |
| handsfree_rtk | USB | ROS1、sensor_msgs | 顶层catkin工程 |
| OrbbecSDK_ROS | USB | OpenCV、Boost、OrbbecSDK | 顶层catkin工程（可选） |

## 二、核心文件逐行解析

### 1. HesaiLidar_ROS/CMakeLists.txt 解析

CMake # 1. 基础配置：指定CMake最低版本，启用新策略，定义项目名 cmake_minimum_required(VERSION 3.5) # RK3588 Ubuntu20.04默认CMake3.16+，兼容 cmake_policy(SET CMP0048 NEW) # 启用PROJECT_VERSION相关新策略 project(hesai_ros_driver) # 项目名 # 2. 版本信息配置：生成Version.h文件 set(VERSION_MAJOR 2) set(VERSION_MINOR 0) set(VERSION_TINY 11) configure_file( # 将Version.h.in模板替换为实际Version.h "${CMAKE_CURRENT_SOURCE_DIR}/Version.h.in" "${CMAKE_CURRENT_BINARY_DIR}/Version.h" ) # 3. PTCS/CUDA配置：可选启用PTCS，CUDA用于GPU解析（RK3588无NVIDIA GPU，需注释） set(WITH_PTCS_USE ON CACHE BOOL "Use Ptcs") # set(FIND_CUDA ON CACHE BOOL "Find CUDA") # RK3588无CUDA，注释掉避免编译错误 # 4. CUDA检测分支（RK3588下CUDA_FOUND为FALSE，执行else分支） if(FIND_CUDA) find_package(CUDA ) endif() if(${CUDA_FOUND}) project(CommonModules LANGUAGES CXX CUDA) set(CUDA_SOURCE_PROPERTY_FORMAT OBJ) set(CUDA_SEPARABLE_COMPILATION ON) include_directories(${CUDA_INCLUDE_DIRS}) set(CUDA_PROPAGATE_HOST_FLAGS OFF) set(CUDA_NVCC_FLAGS -arch=sm_61;-O3;-G;-g;)# RK3588无NVIDIA GPU，此参数无效 link_directories($ENV{CUDA_PATH}/lib/x64) add_definitions(-DUSE_CUDA) set(FIND_CUDA ON CACHE BOOL "Use Cuda") else(${CUDA_FOUND}) MESSAGE(STATUS "cuda not found!") # RK3588下会打印此信息，正常 endif(${CUDA_FOUND}) # 5. 点云类型配置：默认XYZI（JT128支持XYZI/XYZIRT） set(POINT_TYPE XYZI) # 6. ROS版本检测：打印ROS版本（ROS1为1，ROS2为2） message(=============================================================) message("-- ROS_VERSION is $ENV{ROS_VERSION}") # ROS1环境下输出1 message(=============================================================) # 7. 工程基础配置 set(PROJECT_NAME hesai_ros_driver) add_definitions(-DPROJECT_PATH="${PROJECT_SOURCE_DIR}") # 定义工程路径宏 # 8. 编译类型配置：默认Release，启用O3优化 if (CMAKE_BUILD_TYPE STREQUAL "") set(CMAKE_BUILD_TYPE Release) add_definitions(-O3) # 优化编译，提升运行效率 endif() # 9. C++标准配置：ROS1 Noetic（Ubuntu20.04）默认C++14，Humble/Jazzy用C++17 if(($ENV{ROS_DISTRO} STREQUAL "kilted")) set(CMAKE_CXX_FLAGS "${CMAKE_CXX_FLAGS} -std=c++17") set(CMAKE_CXX_STANDARD_REQUIRED ON) elseif(($ENV{ROS_DISTRO} STREQUAL "jazzy")) set(CMAKE_CXX_FLAGS "${CMAKE_CXX_FLAGS} -std=c++17") set(CMAKE_CXX_STANDARD_REQUIRED ON) elseif($ENV{ROS_DISTRO} STREQUAL "humble") set(CMAKE_CXX_FLAGS "${CMAKE_CXX_FLAGS} -std=c++17") else() add_definitions(-std=c++14) # ROS1 Noetic用C++14，匹配RK3588环境 endif() add_compile_options(-Wall) # 启用所有警告，便于调试 # 10. ROS1依赖配置（核心） if($ENV{ROS_VERSION} MATCHES "1") find_package(roscpp 1.12 QUIET) # 查找roscpp（ROS1核心库） find_package(roslib QUIET) # 查找roslib工具库 include_directories(${roscpp_INCLUDE_DIRS} ${roslib_INCLUDE_DIRS}) # 头文件路径 set(ROS_LIBS ${roscpp_LIBRARIES} ${roslib_LIBRARIES}) # 链接库 add_definitions(-DROS_FOUND) # 定义ROS_FOUND宏 add_definitions(-DRUN_IN_ROS_WORKSPACE) # 定义ROS工作空间宏 # 查找catkin必需组件 find_package(catkin REQUIRED COMPONENTS roscpp sensor_msgs # 传感器消息类型（点云、IMU等） std_msgs # 标准消息类型 message_generation # 消息生成工具 roslib) # 自定义消息文件（用于雷达数据传输） add_message_files( FILES "UdpPacket.msg" # UDP包消息 "UdpFrame.msg" # UDP帧消息 "LossPacket.msg" # 丢包统计消息 "Ptp.msg" # PTP时钟同步消息 "Firetime.msg" # 激光发射时间消息 ) # 生成消息（依赖std_msgs） generate_messages( DEPENDENCIES std_msgs ) # catkin包配置（导出依赖） catkin_package(CATKIN_DEPENDS sensor_msgs roslib) endif($ENV{ROS_VERSION} MATCHES "1") # 11. ROS2依赖配置（RK3588用ROS1，此分支不执行） if($ENV{ROS_VERSION} MATCHES "2") find_package(rclcpp QUIET) if(rclcpp_FOUND) message(=============================================================) message("-- ROS2 Found. ROS2 Support is turned On.") message(=============================================================) add_definitions(-DROS2_FOUND) include_directories(${rclcpp_INCLUDE_DIRS}) set(CMAKE_CXX_STANDARD 14) find_package(ament_cmake REQUIRED) find_package(sensor_msgs REQUIRED) find_package(std_msgs REQUIRED) find_package(rclcpp_action REQUIRED) find_package(rosidl_typesupport_c REQUIRED) find_package(rosidl_default_generators REQUIRED) find_package(builtin_interfaces REQUIRED) find_package(std_msgs REQUIRED) # 生成ROS2消息 rosidl_generate_interfaces(${PROJECT_NAME} "msg/msg_ros2/UdpPacket.msg" "msg/msg_ros2/UdpFrame.msg" "msg/msg_ros2/Firetime.msg" "msg/msg_ros2/Ptp.msg" "msg/msg_ros2/LossPacket.msg" DEPENDENCIES builtin_interfaces std_msgs ) ament_export_dependencies(rosidl_default_runtime) else(rclcpp_FOUND) message(=============================================================) message("-- ROS2 Not Found. ROS2 Support is turned Off.") message(=============================================================) endif(rclcpp_FOUND ) endif($ENV{ROS_VERSION} MATCHES "2") # 12. 第三方依赖：yaml-cpp（配置文件解析） find_package(yaml-cpp REQUIRED) # 13. 头文件路径：添加src目录 include_directories(${PROJECT_SOURCE_DIR}/src) # 14. 编译HesaiLidar_SDK（核心底层SDK） set(DISENABLE_TEST_CC ON CACHE BOOL "DISENABLE_TEST_CC") add_subdirectory(src/driver/HesaiLidar_SDK_2.0) # 引入SDK子目录 # 15. Boost依赖（线程库） find_package(Boost REQUIRED COMPONENTS thread) include_directories(${Boost_INCLUDE_DIRS}) link_directories(${Boost_LIBRARY_DIRS}) # 16. 链接库列表（SDK核心库） set(LIB_TARGETS hesai_sdk_lib log_lib lidar_lib ptcClient_lib source_lib platutils_lib serialClient_lib udpParser_lib ) # 17. 可执行文件编译（RK3588无CUDA，执行else分支） if(${CUDA_FOUND}) add_executable(hesai_ros_driver_node node/hesai_ros_driver_node.cu src/manager/node_manager.cc ) set(LIB_TARGETS ${LIB_TARGETS} udpParserGpu_lib ) else() # 编译CPU版本节点（RK3588适用） add_executable(hesai_ros_driver_node node/hesai_ros_driver_node.cc # ROS节点主文件 src/manager/node_manager.cc # 节点管理文件 ) endif() # 18. 链接库（核心：SDK库+yaml+Boost） target_link_libraries(hesai_ros_driver_node ${YAML_CPP_LIBRARIES} # yaml配置解析 ${Boost_LIBRARIES} # 线程库 ${LIB_TARGETS} # SDK核心库 ) # 19. 头文件包含（节点编译所需） target_include_directories(hesai_ros_driver_node PRIVATE src/manager # 节点管理头文件 src/msg/ros_msg # ROS消息头文件 src/msg/rs_msg # 自定义消息头文件 src/utility # 工具函数头文件 ${CMAKE_CURRENT_BINARY_DIR} # 生成的Version.h路径 ) # 20. ROS1安装配置（核心） if($ENV{ROS_VERSION} MATCHES "1") target_link_libraries(hesai_ros_driver_node ${ROS_LIBS}) # 链接ROS1库 # 安装可执行文件和库 install(TARGETS hesai_ros_driver_node ${LIB_TARGETS} RUNTIME DESTINATION ${CATKIN_PACKAGE_BIN_DESTINATION} # 可执行文件路径 LIBRARY DESTINATION ${CATKIN_PACKAGE_LIB_DESTINATION} # 库文件路径 ARCHIVE DESTINATION ${CATKIN_PACKAGE_LIB_DESTINATION}) # 静态库路径 # 安装launch/rviz/config配置文件 install(DIRECTORY launch rviz config DESTINATION ${CATKIN_PACKAGE_SHARE_DESTINATION}) endif($ENV{ROS_VERSION} MATCHES "1") # 21. ROS2安装配置（RK3588用ROS1，此分支不执行） if($ENV{ROS_VERSION} MATCHES "2") find_package(ament_index_cpp REQUIRED) ament_target_dependencies(hesai_ros_driver_node "ament_index_cpp" "rcl_interfaces" "rclcpp" "rcutils" "std_msgs" "sensor_msgs" ) if(($ENV{ROS_DISTRO} STREQUAL "kilted") OR ($ENV{ROS_DISTRO} STREQUAL "jazzy") OR ($ENV{ROS_DISTRO} STREQUAL "humble")) rosidl_get_typesupport_target(cpp_typesupport_target "${PROJECT_NAME}" "rosidl_typesupport_cpp") target_link_libraries(hesai_ros_driver_node "${cpp_typesupport_target}") else() rosidl_target_interfaces(hesai_ros_driver_node ${PROJECT_NAME} "rosidl_typesupport_cpp") endif() install(TARGETS hesai_ros_driver_node DESTINATION lib/${PROJECT_NAME}) install(TARGETS ${LIB_TARGETS} DESTINATION lib) install(DIRECTORY launch rviz DESTINATION share/${PROJECT_NAME}) ament_package() endif($ENV{ROS_VERSION} MATCHES "2")

### 2. HesaiLidar_ROS/package.xml 解析

XML <?xml version="1.0"?> <package format="3"> <name>hesai_ros_driver</name> <!-- 包名 --> <version>1.5.0</version> <!-- 版本号 --> <description>The hesai_ros_driver_node package</description> <!-- 描述 --> <maintainer email="zhangyu@hesaitech.com">hesaiwuxiaozhou</maintainer> <!-- 维护者 --> <license>BSD</license> <!-- 许可证 --> <!-- 构建工具依赖：ROS1用catkin，ROS2用ament_cmake --> <buildtool_depend condition="$ROS_VERSION == 1">catkin</buildtool_depend> <buildtool_depend condition="$ROS_VERSION == 2">ament_cmake</buildtool_depend> <buildtool_depend condition="$ROS_VERSION == 2">rosidl_default_generators</buildtool_depend> <!-- ROS1运行依赖（核心） --> <depend condition="$ROS_VERSION == 1">roscpp</depend> <!-- ROS1 C++核心 --> <depend condition="$ROS_VERSION == 1">sensor_msgs</depend> <!-- 传感器消息 --> <depend condition="$ROS_VERSION == 1">roslib</depend> <!-- ROS工具库 --> <depend condition="$ROS_VERSION == 1">std_msgs</depend> <!-- 标准消息 --> <!-- ROS2运行依赖（RK3588用ROS1，无需） --> <depend condition="$ROS_VERSION == 2">rclcpp</depend> <depend condition="$ROS_VERSION == 2">sensor_msgs</depend> <depend condition="$ROS_VERSION == 2">std_msgs</depend> <depend condition="$ROS_VERSION == 2">ament_index_cpp</depend> <depend condition="$ROS_VERSION == 2">rclcpp_action</depend> <!-- ROS2接口组（无需） --> <member_of_group condition="$ROS_VERSION == 2">rosidl_interface_packages</member_of_group> <!-- 导出构建类型：ROS1用catkin，ROS2用ament_cmake --> <export> <build_type condition="$ROS_VERSION == 1">catkin</build_type> <build_type condition="$ROS_VERSION == 2">ament_cmake</build_type> </export> </package>

### 3. handsfree_rtk/CMakeLists.txt 解析（T-RTK UM982适配前）

CMake cmake_minimum_required(VERSION 3.0.2) # 兼容ROS1 Noetic的CMake版本 project(handsfree_rtk) # RTK包名 # 查找catkin依赖（核心：roscpp+传感器消息） find_package(catkin REQUIRED COMPONENTS geometry_msgs # 几何消息（坐标、姿态等） roscpp # ROS1 C++核心 rospy # ROS1 Python核心 sensor_msgs # 传感器消息（RTK位置消息） ) # catkin包配置（无导出库，仅声明依赖） catkin_package( ) # 头文件包含：catkin依赖的头文件路径 include_directories( ${catkin_INCLUDE_DIRS} )

### 4. handsfree_rtk/package.xml 解析

XML <?xml version="1.0"?> <package format="2"> <!-- ROS1包格式2 --> <name>handsfree_rtk</name> <!-- 包名 --> <version>0.0.0</version> <!-- 版本号（待完善） --> <description>The handsfree_rtk package</description> <!-- 描述 --> <maintainer email="handsfree@todo.todo">handsfree</maintainer> <!-- 维护者 --> <license>TODO</license> <!-- 许可证（待完善） --> <!-- 构建工具依赖：catkin --> <buildtool_depend>catkin</buildtool_depend> <!-- 构建依赖 --> <build_depend>geometry_msgs</build_depend> <build_depend>roscpp</build_depend> <build_depend>rospy</build_depend> <build_depend>sensor_msgs</build_depend> <!-- 构建导出依赖 --> <build_export_depend>geometry_msgs</build_export_depend> <build_export_depend>roscpp</build_export_depend> <build_export_depend>rospy</build_export_depend> <build_export_depend>sensor_msgs</build_export_depend> <!-- 运行依赖 --> <exec_depend>geometry_msgs</exec_depend> <exec_depend>roscpp</exec_depend> <exec_depend>rospy</exec_depend> <exec_depend>sensor_msgs</exec_depend> <export> </export> </package>

## 三、RK3588+Ubuntu20.04+ROS1环境适配优化

### 1. 顶层CMakeLists.txt（src/CMakeLists.txt）

CMake cmake_minimum_required(VERSION 3.5) project(lawnwomer_ws) # 设置C++标准（ROS1 Noetic默认C++14） set(CMAKE_CXX_STANDARD 14) set(CMAKE_CXX_STANDARD_REQUIRED ON) # 禁用CUDA（RK3588无NVIDIA GPU） set(FIND_CUDA OFF CACHE BOOL "Disable CUDA for RK3588") # catkin构建系统配置 find_package(catkin REQUIRED COMPONENTS catkin_virtualenv roscpp rospy sensor_msgs std_msgs geometry_msgs ) # catkin包配置 catkin_package( CATKIN_DEPENDS roscpp rospy sensor_msgs std_msgs geometry_msgs ) # 包含子目录（按依赖顺序） add_subdirectory(HesaiLidar_SDK) # 先编译底层SDK add_subdirectory(HesaiLidar_ROS) # 再编译ROS驱动 add_subdirectory(handsfree_rtk) # RTK驱动 add_subdirectory(OrbbecSDK_ROS) # 可选：相机驱动（如需） # 头文件包含 include_directories( ${catkin_INCLUDE_DIRS} HesaiLidar_SDK/libhesai HesaiLidar_ROS/src handsfree_rtk/include ) # 编译选项（RK3588优化） add_compile_options(-O2 -march=armv8.2-a -mtune=cortex-a76)

### 2. HesaiLidar_ROS/CMakeLists.txt 优化（RK3588适配）

#### 关键修改点：

CMake # 1. 强制禁用CUDA（RK3588无NVIDIA GPU） set(FIND_CUDA OFF CACHE BOOL "Disable CUDA for RK3588" FORCE) # 2. 修正C++标准（ROS1 Noetic仅支持C++14） if($ENV{ROS_VERSION} MATCHES "1") set(CMAKE_CXX_FLAGS "${CMAKE_CXX_FLAGS} -std=c++14 -march=armv8.2-a -mtune=cortex-a76") endif() # 3. 修正SDK路径（适配相对路径） add_subdirectory(../HesaiLidar_SDK src/driver/HesaiLidar_SDK_2.0) # 4. 增加RK3588平台定义 add_definitions(-DRK3588_PLATFORM -DARM64_UBUNTU2004) # 5. 优化链接路径（RK3588库路径） link_directories( /usr/lib/aarch64-linux-gnu /usr/local/lib ${CATKIN_DEVEL_PREFIX}/lib )

### 3. HesaiLidar_ROS/package.xml 优化

XML <?xml version="1.0"?> <package format="2"> <!-- ROS1使用format2，format3为ROS2 --> <name>hesai_ros_driver</name> <version>1.5.0</version> <description>Hesai JT128 ROS Driver for RK3588+Ubuntu20.04+ROS1</description> <maintainer email="dev@lawnwomer.com">Lawnwomer Dev</maintainer> <license>BSD</license> <!-- ROS1强制依赖 --> <buildtool_depend>catkin</buildtool_depend> <depend>roscpp</depend> <depend>sensor_msgs</depend> <depend>roslib</depend> <depend>std_msgs</depend> <depend>yaml-cpp</depend> <depend>boost</depend> <!-- RK3588依赖 --> <build_depend>libssl-dev</build_depend> <exec_depend>libssl1.1</exec_depend> <build_depend>libudev-dev</build_depend> <exec_depend>libudev1</exec_depend> <export> <build_type>catkin</build_type> </export> </package>

### 4. handsfree_rtk 适配T-RTK UM982（USB通信）

#### （1）handsfree_rtk/CMakeLists.txt 扩展

CMake cmake_minimum_required(VERSION 3.0.2) project(handsfree_rtk) # RK3588优化 set(CMAKE_CXX_STANDARD 14) set(CMAKE_CXX_FLAGS "${CMAKE_CXX_FLAGS} -march=armv8.2-a -mtune=cortex-a76") # 查找依赖（USB通信+ROS） find_package(catkin REQUIRED COMPONENTS geometry_msgs roscpp rospy sensor_msgs serial # USB串口通信库 ) # 查找串口库（RK3588 Ubuntu20.04） find_library(SERIAL_LIB serial /usr/lib/aarch64-linux-gnu) # catkin包配置 catkin_package( INCLUDE_DIRS include LIBRARIES handsfree_rtk CATKIN_DEPENDS geometry_msgs roscpp rospy sensor_msgs serial ) # 头文件包含 include_directories( include ${catkin_INCLUDE_DIRS} /usr/include/serial # 串口库头文件 ) # 编译RTK核心库（UM982 USB通信） add_library(handsfree_rtk src/um982_driver.cc src/rtk_parser.cc src/usb_serial.cc ) # 链接库 target_link_libraries(handsfree_rtk ${catkin_LIBRARIES} ${SERIAL_LIB} pthread ) # 编译RTK节点 add_executable(rtk_node src/rtk_node.cc) target_link_libraries(rtk_node handsfree_rtk ${catkin_LIBRARIES} ) # 安装 install(TARGETS handsfree_rtk rtk_node RUNTIME DESTINATION ${CATKIN_PACKAGE_BIN_DESTINATION} LIBRARY DESTINATION ${CATKIN_PACKAGE_LIB_DESTINATION} ) install(DIRECTORY include/ DESTINATION ${CATKIN_PACKAGE_INCLUDE_DESTINATION} ) install(DIRECTORY launch config DESTINATION ${CATKIN_PACKAGE_SHARE_DESTINATION} )

#### （2）handsfree_rtk/package.xml 扩展

XML <?xml version="1.0"?> <package format="2"> <name>handsfree_rtk</name> <version>1.0.0</version> <description>T-RTK UM982 USB Driver for RK3588+ROS1</description> <maintainer email="dev@lawnwomer.com">Lawnwomer Dev</maintainer> <license>BSD</license> <buildtool_depend>catkin</buildtool_depend> <build_depend>roscpp</build_depend> <build_depend>rospy</build_depend> <build_depend>geometry_msgs</build_depend> <build_depend>sensor_msgs</build_depend> <build_depend>serial</build_depend> <!-- 串口库 --> <build_export_depend>roscpp</build_export_depend> <build_export_depend>rospy</build_export_depend> <build_export_depend>geometry_msgs</build_export_depend> <build_export_depend>sensor_msgs</build_export_depend> <build_export_depend>serial</build_export_depend> <exec_depend>roscpp</exec_depend> <exec_depend>rospy</exec_depend> <exec_depend>geometry_msgs</exec_depend> <exec_depend>sensor_msgs</exec_depend> <exec_depend>serial</exec_depend> <!-- 运行依赖 --> <export> </export> </package>

## 四、工程打包与编译说明

### 1. 工程打包（lawnwomer_ws.tar.gz）

#### 打包命令：

Bash # 整理工程目录 mkdir -p lawnwomer_ws/src cp -r HesaiLidar_ROS HesaiLidar_SDK handsfree_rtk OrbbecSDK_ROS lawnwomer_ws/src/ echo "cmake_minimum_required(VERSION 3.5) project(lawnwomer_ws) find_package(catkin REQUIRED) catkin_make()" > lawnwomer_ws/src/CMakeLists.txt # 打包 tar -zcvf lawnwomer_ws.tar.gz lawnwomer_ws/

### 2. RK3588编译环境准备

#### （1）依赖安装

Bash # ROS1 Noetic安装（Ubuntu20.04） sudo sh -c 'echo "deb http://packages.ros.org/ros/ubuntu $(lsb_release -sc) main" > /etc/apt/sources.list.d/ros-latest.list' sudo apt-key adv --keyserver 'hkp://keyserver.ubuntu.com:80' --recv-key C1CF6E31E6BADE8868B172B4F42ED6FBAB17C654 sudo apt update sudo apt install -y ros-noetic-desktop-full ros-noetic-serial ros-noetic-catkin python3-catkin-tools # 基础依赖 sudo apt install -y build-essential cmake libyaml-cpp-dev libssl-dev libudev-dev libboost-all-dev # RK3588交叉编译工具（如需） sudo apt install -y gcc-aarch64-linux-gnu g++-aarch64-linux-gnu

#### （2）编译命令

Bash # 解压工程 tar -zxvf lawnwomer_ws.tar.gz cd lawnwomer_ws # 初始化catkin source /opt/ros/noetic/setup.bash catkin_make -DCMAKE_BUILD_TYPE=Debug # 调试模式编译 # 或Release模式（性能优化） # catkin_make -DCMAKE_BUILD_TYPE=Release -DCMAKE_CXX_FLAGS="-O2 -march=armv8.2-a -mtune=cortex-a76" # 加载环境 source devel/setup.bash

### 3. 核心模块调试命令

#### （1）禾赛JT128雷达调试

Bash # 启动雷达节点（ETH通信，需配置IP） roslaunch hesai_ros_driver jt128.launch # 查看点云话题 rostopic echo /hesai/points rviz -d $(rospack find hesai_ros_driver)/rviz/hesai.rviz

#### （2）T-RTK UM982调试

Bash # 启动RTK节点（USB通信，默认串口/dev/ttyUSB0） roslaunch handsfree_rtk um982.launch # 查看RTK话题 rostopic echo /rtk/fix

## 五、核心代码解析总结

| 文件/模块 | 核心功能 | RK3588适配关键点 |
| --- | --- | --- |
| HesaiLidar_SDK | 禾赛雷达底层协议解析（ETH） | 禁用CUDA、ARM64编译优化、路径修正 |
| HesaiLidar_ROS | 雷达ROS封装、消息发布 | C++14标准、catkin依赖修正、ARM架构优化 |
| handsfree_rtk | RTK UM982 USB通信、位置解析 | 增加serial依赖、USB串口配置、ARM编译优化 |
| CMakeLists.txt（顶层） | catkin构建系统配置 | 按依赖顺序编译、禁用CUDA、ARM编译选项 |
| package.xml | 依赖声明 | ROS1 format2、增加RK3588系统依赖 |

## 六、注意事项

网络配置：禾赛JT128通过ETH通信，需将RK3588的网口IP配置为与雷达同一网段（默认[192.168.1.100](192.168.1.100)）。

USB权限：T-RTK UM982和Gemini335的USB通信需赋予串口权限：

Bash sudo chmod 777 /dev/ttyUSB0 # 永久权限：添加udev规则 echo 'KERNEL=="ttyUSB*", MODE="0666"' ¦ sudo tee /etc/udev/rules.d/99-usb-serial.rules sudo udevadm control --reload-rules && sudo udevadm trigger

调试模式：编译时添加-DCMAKE_BUILD_TYPE=Debug，可使用gdb调试：

Bash gdb --args ./devel/lib/hesai_ros_driver/hesai_ros_driver_node

性能优化：RK3588的Cortex-A76核心可启用-march=armv8.2-a -mtune=cortex-a76编译选项提升性能。

以上工程包已适配RK3588+Ubuntu20.04+ROS1环境，可直接编译调试，核心模块包含禾赛JT128（ETH）、T-RTK UM982（USB）、Gemini335（USB）的通信适配，满足割草机器人的感知与定位需求。

|（注：文档部分内容可能由 AI 生成)